Error Tolerant Adder Type-1 Application in Facial Recognition

O V Gnana Swathika, V S Hitesh, David Alappat.

#### 1School of Electrical Engineering, Vellore Institute of Technology, Chennai, India

#### 2*Centre of Smart Grid Technologies, Vellore Institute of Technology, Chennai, India

*gnanaswathika.ov@vit.ac.in

Abstract:

The implementation of deep neural networks on hardware constrained edge devices causes a major downfall in terms of power consumption and propagation delay. To address this issue, we are trying to implement Error Tolerant Adder type-1 to a software based facial recognition system. We have actually used an 8-bit hybrid ETA-1 adder, during the Manhattan distance calculation, which is important for comparing the extracted facial features with the one stored in the system database. The implementation showed comparable results such that energy consumption decreased by 42.3%, propagation delay improved by 41.3% (comparing with a Standard Ripple Carry Adder); with only a marginal 1.7% trade-off with accuracy. Since we can implement approximate computing only to error resilient systems this trade off is negligible compared to the gains we achieve.

Keywords:

Deep Neural Networks, Error Tolerant Type Adder type-1, Facial Recognition, Manhattan Distance, Approximate Computing, Error-Resilient System.

1.Introduction:

The rapid development of artificial intelligence has caused widespread deployment of Complex Neural Networks over hardware. But these networks demand large amount of hectic calculation which when implemented using the conventional accurate adders leads to heavy power consumption and propagation delay. This bottleneck, whereby increasing accuracy leads to greater efficiency losses has led to the introduction of approximate computing. Perfect mathematical accuracy is not necessary in fields such as neural networks which inherently possess error resilience and still can recognize patterns to a great extent. Therefore, it is a strategic move to implement approximation computing in such areas and hence achieve more gains with as little expense. Though there are a lot of approximation logic proposed recently in different areas of computation, in this paper we have used one of the most widely used approximation logic, ETA type 1, logic within our software framework. This paper then evaluates the tradeoffs and gains we achieved through its demonstration.

2.Methodology:

The incorporation of software-modeled approximate arithmetic units into the classification layer of a Convolutional Neural Network for facial recognition and evaluate the tradeoff between computational accuracy and hardware efficiency, is the core focus of this study.

The software environment made replaces standard exact accumulation units with an ETA type-1 hybrid approximation variant to compute the similarity metrics of facial embeddings. To demonstrate the working of the hybrid adder used we have made a interactive UI interface. The workflow is provided in Figure 1.

Below is a link to demonstrate the working of the ETA Type-1 in online: https://github.com/Hitesh070/My-Projects/blob/main/approximate_adder_ui.html

2.1 Simulated Hybrid Adder Architecture

The 8-bit hybrid approximate adder within the software model is divided into two parts. The first 4-bits belong to accurate part and the other to the inaccurate part.

The higher order 4 bits of operands belonging to the accurate part are processed using a conventional precise adder block. This is done to ensure that no high magnitude errors are made which can potentially lead to greater biometric irregularities. On the other hand, the lower 4-bits are processed using the hybrid adder with ETA type-1 logic. This configuration cuts the carry-propagation chain at the 4th bit, thus eliminating the critical path delay and boosting computational speed.

The final hybrid sum is mathematically expressed as:

SumHybrid=SumMSB_Exact×24+SumLSB_Approx

  - System Architecture and Algorithm Implementation

The biometric verification phase has a pre-trained CNN architecture designed to map face images into a dense 512-dimensional space. The simulated 8-bit hybrid approximate adder is deployed in the final stage, preserving macroscopic accuracy. The execution pipeline is divided into three execution phases:

- Feature Extraction Phase: The system utilizes the pre-trained deep learning model to process a live facial image captured by the camera, generating an array of 512 numerical features. For every authentication attempt, the system pairs this live feature vector (L) with a reference feature vector (R) from the authorized user database.

- Approximate Distance Calculation: To determine the mathematical similarity between the two 512-dimensional arrays, the system computes the absolute difference between each corresponding feature element and continuously accumulates them to find the total Manhattan Distance (L1 Norm). This is defined mathematically as:

DL,R=i=1512Li-Ri

where i represent each index of the 512-dimensional array. This repetitive accumulation loop represents the most computationally demanding phase. Within our Python simulation environment, the standard precise addition operator (+) within this loop is entirely substituted by the proposed 8-bit hybrid ETA-1 adder function

- Classification and Thresholding: Once the approximate Manhattan distance is calculated it is evaluated against a predefined biometric verification threshold from the database. If the value falls within the threshold, the system registers a positive registration and if it is above; as a negative registration.

2.3 Evaluation Setup

The performance of the proposed architecture is verified across two broad experimental domains:

- Arithmetic Error Metrics: To quantitatively measure the error produced by the proposed adder, it is subjected to all 65,536 input combinations possible for an 8-bit dual-operand system. The mathematical deviation is evaluated using standard VLSI metrics such as Error Rate (ER), Error Distance (ED), Mean Error Distance (MED), and Normalized Mean Error Distance (NMED).

- Hardware Efficiency Metrics: The physical benefits of the approximate hybrid adder used is analyzed using simulated energy consumption per addition (measured in picojoules, pJ), critical path propagation delay (measured in nanoseconds, ns), and overall silicon footprint (quantified via total gate count). These metrics are compared against a standard 8-bit precise Ripple Carry Adder baseline to determine the final Power-Delay Product (PDP) optimizations.

Figure 1: Block Diagram

3.Results and Discussion:

Comparing a device that uses a standard precise Ripple Carry Adder (RCA), we have analyzed the performance metrices of our software modeled 8-bit Hybrid Approximate System. The discussions are divided into two domains: The first one is the Hardware Efficiency and simplification provided by the model compared to a standard Ripple Carry Adder (RCA), and the second one is based on the Biometric Accuracy and Error Analysis.

3.1 Hardware Efficiency and Architectural Simplification

- Resource Optimization: For the optimization of the device, we replaced the standard carry propagation logic cells in the lower 4 bits with a non-truncating gating structure of the ETA-1 logic, the simulation results are provided in Fig 2, which provides us with various information like the power consumption of the device, propagation delay, silicon area, and the system accuracy which is very important to analyze the performance of the device, which is provided in Fig 3.

- The theoretical gate count dropped from 96 gates for a standard RCA to just 58 gates for the software modeled 8-bit Hybrid Approximate System. This physical change in the number of transistors used in the design brings us a 40% reduction in the total silicon area. This area is critical and can be utilized in the production of biometric systems which have space constraint on System on Chip (SoC) edge devices.

Figure 2: Hardware Resource Overhead Comparison

Figure 3: System Accuracy

- Energy Efficiency: The dynamic power dissipation in a system is determined by the number of transistor switching activity in that particular system. Since the non-truncating gating structure of the ETA-1 logic of the 4th bit boundary uses lesser number of switching of transistors than a standard RCA, from the simulations we found out that power consumption has reduced by 42.34%, the values dropped from 12.40 pJ to 7.15 pJ per addition under active workload simulation. The energy consumption chart is provided in Fig 4.

- Speed and Latency: The elimination of the lower order carry path plays an important role in the delay of the system. The software modeled 8-bit Hybrid Approximate System has achieved a 41.3% of improvement in the propagation delay by reducing the latency from 1.45 ns to just 0.85 ns. This helps the biometric system to operate at a higher clock frequency or maintain a standard output when the supply voltage is reduced to maximize the efficiency. The propagation delay is already mentioned in Fig 2.

Figure 4: Energy Consumption

3.2 Biometric Accuracy and Arithmetic Error Analysis

To evaluate the performance metrices of the software modeled 8-bit Hybrid Approximate System, we subjected the system for a simulation with 65,536 input permutations. The resulting error metrics are compiled in Table 1.

The feasibility of the system is determined by the Mean Error Distance (MED) which is restricted to 3.20. Because the exact addition on the MSB is perfect and the maximum possible error distance is strictly defined, thus ensuring that the high error magnitude arithmetic faults do not occur. To visualize this, we have plotted an error distance map and displayed it in Fig 5.

When we integrated the software modeled 8-bit Hybrid Approximate System into the actual deep Convolutional Neural Network (CNN) System to implement the biometric device, while using the standard precise arithmetic adder the system had an efficiency of 98.5% but after implementing the Type-I ETA logic it marginally dropped to 96.8%. The system accuracy has already been discussed in Fig 3. Also, another important performance metrices is hardware propagation delay, which is represented in Fig 6.

Table 1: Performance metrices

| Metric | Exact Adder (Normal) | Approximate Adder (ETA-I) |
| --- | --- | --- |
| Error Rate (ER) | 0.00% | 68.36% |
| Max Error Distance (ED) | 0.0 | 15.0 |
| Mean Error Distance (MED) | 0.0000 | 3.1367 |
| Normalized MED (NMED) | 0.000000 | 0.006150 |
| Energy Consumption (pJ) | 12.40 | 7.15 |
| Propagation Delay (ns) | 1.45 | 0.85 |

Figure 5: Error Distance Map

Figure 6: Hardware Propagation Delay

This minimal drop 1.7% drop in accuracy demonstrates the resilience of the developed CNN, with less power and silicon area consumption it can provide decent performance as similar as to a CNN that uses a precise arithmetic adder.

4. Conclusion:

This study shows us that implementing an approximate arithmetic system with a Deep Learning system is a highly effective way to reduce energy consumption and silicon area on an edge device where these two factors play a crucial role in the design of the SoC design.

The importance of this architectural change in the SoC design is highlighted using a heavy computational demand of a biometric verification on an edge device. Comparing 512-dimensional facial embeddings natively on an edge device requires massive repetitive addition operations. Since most of the operations on the edge device operate on simple arithmetic operations the best way to increase the system’s efficiency is to implement a Type – I ETA approximation logic, where we used an 8-bit number and split it into two parts. In the 4-bits of the MSB part we use an exact Ripple Carry Adder (RCA) and in the 4-bits of the LSB part we use an ETA-1 approximation logic. By isolating the carry propagation path to the upper half of the bits, the circuit eliminates the propagation delay that limits traditional arithmetic adders.

The proposed system achieved a biometric system accuracy of 96.8% while facing a marginal decrease of 1.7% from the system that uses an RCA which has an accuracy of 98.5%. This confirms that high level of geometric and statistical facial features is preserved perfectly. Consequently, the hybrid ETA-1 adder functions as an exceptionally powerful acceleration tool for hardware-constrained environments—such as battery-operated attendance scanners, smart doorbells, and portable security cameras—where energy budgets are severely restricted.

In conclusion, this research highlights approximate computing as a crucial design breakthrough for the next generation of energy-efficient systems where errors are tolerable to a extend. For power-limited biometric applications, trading less than 2% of classification accuracy for a massive 42.3% savings in dynamic energy and a 39.5% reduction in gate area represents an incredibly favorable engineering compromise. Future iterations of this work could explore integrating this exact approximate logic into alternative high-dimensional evaluation metrics, such as Cosine Similarity or Euclidean Distance, or mapping the architecture directly onto the parallel Multiply-Accumulate (MAC) arrays within the internal convolutional layers of the neural network itself.

References:

[1] S. Ullah, S. S. Sahoo and A. Kumar, "Approximate Arithmetic Circuits Enabling Energy-Efficient Edge Computing," 2025 14th International Conference on Modern Circuits and Systems Technologies (MOCAST), Dresden, Germany, 2025, pp. 1-6, doi: 10.1109/MOCAST65744.2025.11083734.

[2] D. Cordero-Chavarría, L. G. Leόn-Vega and J. Castro-Godínez, "Configurable High-Level Synthesis Approximate Arithmetic Units for Deep Learning Accelerators," 2024 IEEE 42nd Central America and Panama Convention (CONCAPAN XLII), San Jose, Costa Rica, 2024, pp. 1-6, doi: 10.1109/CONCAPAN63470.2024.10933846.

[3] S. A. K. Gharavi and S. Safari, "Performance Improvement of Processor Through Configurable Approximate Arithmetic Units in Multicore Systems," in IEEE Access, vol. 12, pp. 43907-43917, 2024, doi: 10.1109/ACCESS.2024.3380912.

[4] J. Rogers, T. Soliman and M. Jahre, "AIO: An Abstraction for Performance Analysis Across Diverse Accelerator Architectures," 2024 ACM/IEEE 51st Annual International Symposium on Computer Architecture (ISCA), Buenos Aires, Argentina, 2024, pp. 487-500, doi: 10.1109/ISCA59077.2024.00043.

[5] F. Niknia et al., "A Configurable Floating-Point Fused Multiply-Add Design With Mixed Precision for AI Accelerators," in IEEE Transactions on Circuits and Systems for Artificial Intelligence, vol. 2, no. 3, pp. 248-261, Sept. 2025, doi: 10.1109/TCASAI.2025.3569511.

[6] A. M. Dalloo, A. Jaleel Humaidi, A. K. Al Mhdawi and H. Al-Raweshidy, "Approximate Computing: Concepts, Architectures, Challenges, Applications, and Future Directions," in IEEE Access, vol. 12, pp. 146022-146088, 2024, doi: 10.1109/ACCESS.2024.3467375.

[7] M. Ronchi, S. D. Giacomo, M. Amadori, G. Borghi, M. Carminati and C. Fiorini, "Design, Implementation, and Analysis of an Integrated Switched Capacitor Analog Neuron for Edge Computing AI Accelerators," in IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 72, no. 8, pp. 3947-3960, Aug. 2025, doi: 10.1109/TCSI.2025.3546521.

[8] S. -H. Noh, S. Lee, B. Shin, S. Park, Y. Jang and J. Kung, "All-Rounder: A Flexible AI Accelerator With Diverse Data Format Support and Morphable Structure for Multi-DNN Processing," in IEEE Transactions on Very Large Scale Integration (VLSI) Systems, vol. 33, no. 5, pp. 1264-1277, May 2025, doi: 10.1109/TVLSI.2025.3540346.

[9] D. S. Narayana, C. Jayakumar, R. S, S. Y and B. P. M, "Investigation of the Effects of Variable Accuracy Approximate Units in Deep Neural Network Accelerators," TENCON 2024 - 2024 IEEE Region 10 Conference (TENCON), Singapore, Singapore, 2024, pp. 1260-1263, doi: 10.1109/TENCON61640.2024.10902871.

[10] S. S. Sahoo, S. Ullah, S. Bhattacharjee and A. Kumar, "AxOCS: Scaling FPGA-Based Approximate Operators Using Configuration Supersampling," in IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 71, no. 6, pp. 2646-2659, June 2024, doi: 10.1109/TCSI.2024.3385333.

[11] N. Zhu, W. L. Goh, W. Zhang, K. S. Yeo, and Z. H. Kong, "Design of Low-Power High-Speed Truncation-Error-Tolerant Adder and Its Application in Digital Signal Processing," IEEE Transactions on Very Large Scale Integration (VLSI) Systems, vol. 18, no. 8, pp. 1225-1229, Aug. 2010, doi: 10.1109/TVLSI.2009.2020591.

[12] F. Ahmadi, M. R. Semati, and H. Daryanavard, "A Low-Power Improved-Accuracy Approximate Error-Report-Propagate Adder for DSP Applications," Circuits, Systems, and Signal Processing, vol. 42, pp. 3744-3769, Jan. 2023, doi: 10.1007/s00034-023-02291-9.

[13] S. Xiang et al., "Nonlinear Hopf-based cochlear network: Bio-inspired modeling, analysis, and FPGA implementation for cochlear implant applications," Biomedical Signal Processing and Control, vol. 120, Art. no. 109929, 2026, doi: 10.1016/j.bspc.2026.109929.

[14] A. M. Dalloo and A. J. Humaidi, "Optimizing Machine Learning Models with Data-level Approximate Computing: The Role of Diverse Sampling, Precision Scaling, Quantization and Feature Selection Strategies," Results in Engineering, vol. 24, Art. no. 103451, 2024, doi: 10.1016/j.rineng.2024.103451.

[15] J. Li, X. Cai, Y. Dai, W. Yin and L. Wang, "Toward Efficient Edge AI With Heterogeneous Computing and Multilevel Optimization," in IEEE Transactions on Very Large Scale Integration (VLSI) Systems, 2026, doi: 10.1109/TVLSI.2026.3697086.

[16] P. P. Shah, J. Munjani, D. Vyas and A. Sharma, "Review of Cutting-Edge Latest Trends and Innovations in Low-Power VLSI Physical Design Techniques for Next-Gen AI/ML Hardware," in 2025 5th Asian Conference on Innovation in Technology (ASIANCON), Pimpri, India, 2025, pp. 1-8, doi: 10.1109/ASIANCON66527.2025.11280810.

[17] H. A. Ziad and A. A. Sakib, "HALOC-AxA: An Area/-Energy-Efficient Approximate Adder for Image Processing Application," in 2025 IEEE International Midwest Symposium on Circuits and Systems (MWSCAS), Lansing, MI, USA, 2025.

[18] M. Chen, B. Wang, J. Chen, H. Wang, Y. Liu, and L. Wu, "A 14-bit 3-MS/s SAR ADC with self-calibration for single-event transient mitigation," Microelectronics Reliability, vol. 175, Art. no. 115904, 2025, doi: 10.1016/j.microrel.2025.115904.

[19] Z. Wang, "Multi-Dimensional Schemes for Reliable and Efficient Neural Networks: Analysis, Design, and Evaluation," Ph.D. dissertation, Northeastern Univ., Boston, MA, USA, 2025. [Online]. Available: ProQuest Dissertations & Theses, Order No. 31996572.

[20] M. E. Weingarten, M. Grieco, S. A. Edwards, and T. A. Khan, "Icicle: Open-Source Hardware Support for Top-Down Microarchitectural Analysis on RISC-V," in 2025 IEEE International Symposium on Workload Characterization (IISWC), Irvine, CA, USA, 2025, pp. 464-477.

[21] S. Swetha, B. Bharghavi, B. Vasundara, and N. S. S. Reddy, "High Speed Approximate Adder for Error Tolerant Image Processing Applications," in 2024 4th International Conference on Mobile Networks and Wireless Communications (ICMNWC), Tumkuru, India, 2024, pp. 1-6, doi: 10.1109/ICMNWC63764.2024.10872196.

[22] J. Chen, J. Cai, Y. Zhang, Y. Yin, and B. Tang, "A 1500 V/μs slew rate, 300 MHz GBW operational amplifier in 0.18-μm HV BCD process for CMOS system-level monolithic integration," Microelectronics Journal, vol. 167, Art. no. 106914, 2026, doi: 10.1016/j.mejo.2025.106914.

[23] D. Upadhyay, S. Deliri, A. Mattsson, P. Peltoniemi, and L. Aarniovuori, "Review on Silicon Carbide based High-Fundamental Frequency Inverters for High-Speed Drives," IEEE Access, 2025, doi: 10.1109/ACCESS.2025.3580521.

[24] K. Bhardwaj et al., "Preventing False Activations in Autonomous Vehicles: A Memristive Associative Learning Approach with Selective Sensor Pairing," in 2025 14th International Conference on Modern Circuits and Systems Technologies (MOCAST), Dresden, Germany, 2025, pp. 1-4, doi: 10.1109/MOCAST65744.2025.11083932.

[25] V. Gupta, D. Mohapatra, A. Raghunathan, and K. Roy, "Low-Power Digital Signal Processing Using Approximate Adders," IEEE Transactions on Computer-Aided Design of Integrated Circuits and Systems, vol. 32, no. 1, pp. 124-137, Jan. 2013, doi: 10.1109/TCAD.2012.2217962.

[26] P. Balasubramanian, R. Nayar, D. L. Maskell, and N. E. Mastorakis, "An Approximate Adder With a Near-Normal Error Distribution: Design, Error Analysis and Practical Application," IEEE Access, vol. 9, pp. 4518-4530, 2021, doi: 10.1109/ACCESS.2020.3047651.

[27] N. Van Toan and J. Lee, "FPGA-Based Multi-Level Approximate Multipliers for High-Performance Error-Resilient Applications," IEEE Access, vol. 8, pp. 25481-25497, 2020, doi: 10.1109/ACCESS.2020.2970968.